# Online Examination Network: Setup and Execution Guide

Jackfruit Mini Project (SDN + Socket Programming). Requirements, execution steps and commands in order, covering everything built so far. The source code itself is in the project folder exam-sdn (exam-sdn.zip); this guide lists what each file does and the exact commands to run it.

## 1. Project in brief

Students take a multi-user online exam over TCP sockets. The exam server runs inside a Mininet network controlled by a Ryu (OpenFlow 1.3) controller. The controller monitors traffic, blocks unauthorized hosts, limits background traffic during congestion, and re-routes around a failed link. Students can answer from a terminal client or from a web page (10 questions, 15 minutes). Everything runs inside an Ubuntu virtual machine.

## 2. Requirements

### 2.1 Host computer (Windows)

| Item | Requirement |
| --- | --- |
| Computer | Windows 10 or 11, virtualization (VT-x / AMD-V) enabled in BIOS, about 60 GB free disk, 8 GB RAM or more recommended |
| VirtualBox | Current version, to run the Ubuntu VM |
| Ubuntu ISO | ubuntu-26.04.1-desktop-amd64.iso (any recent Ubuntu works) |
| VS Code | With the Remote - SSH extension (Microsoft). Optional inside the VM: Python, Rainbow CSV |

### 2.2 Virtual machine settings

- Base memory 6144 MB (minimum 4096), 2 to 4 CPUs, 40 GB disk.

- Display: video memory 128 MB, graphics controller VMSVGA, 3D acceleration off.

- Network: NAT with a port-forwarding rule: TCP, host IP 127.0.0.1, host port 2222, guest port 22.

### 2.3 Software inside the VM

| Package | Purpose |
| --- | --- |
| mininet, openvswitch-switch | Virtual network and switches |
| iperf (version 2) | UDP background traffic for congestion tests |
| openssh-server | VS Code connection |
| netcat-openbsd, curl, git, unzip | Utilities |
| python3-matplotlib | Charts from the results |
| Python 3.9 virtual environment (created with uv) | Runs Ryu, because the system Python is too new for Ryu |
| ryu 4.34, eventlet 0.30.2, setuptools below 58, wheel, pbr | Controller framework and its build tools |
| Firefox (included with the Ubuntu desktop) | Web exam |

### 2.4 Project files

| File | What it does |
| --- | --- |
| common/protocol.py | Length-prefixed JSON messages over TCP |
| server/exam_server.py | Threaded TCP exam server: roster check, questions, answers, timer, auto-submit, scoring, events to the controller. The option --per-student gives each student their own timer (web mode) |
| client/student_client.py | Student client: automatic random answers for experiments, or --interactive to type answers; logs response times |
| web/gateway.py and web/index.html | Web exam: the page talks to the gateway, which opens one real TCP connection per student to the exam server |
| controller/exam_controller.py and controller/netgraph.py | Ryu controller: topology discovery, shortest-path routing, monitoring, congestion limit, link-failure reroute, blocking |
| topology/exam_topo.py | Mininet topology: 4 switches (5 with the backup path), 12 hosts, 10 Mbps bottleneck between s1 and s4 |
| tests/run_experiment.py | One experiment run (students, background load, intruder, link failure) |
| tests/analyze.py | Latency, completion and fairness numbers for one run |
| tests/run_matrix.py and tests/plot.py | Whole evaluation set with repeats, then charts |
| tests/web_demo.py | Starts the web exam inside Mininet |
| data/questions.json, data/roster.txt | The 10 questions and the allowed student IDs (S001 to S040) |

## 3. One-time setup

### 3.1 Create the virtual machine

- In VirtualBox click New. Name it exam-vm and choose the Ubuntu ISO (if it is not listed, use the dropdown, choose Other, and browse to the file). Tick Skip Unattended Installation.

- Set 6144 MB memory, 2 or more CPUs, and a 40 GB disk.

- Before starting, open Settings, then Display (video memory 128 MB, VMSVGA, 3D off), then Network, Adapter 1, Advanced, Port Forwarding, and add the rule from section 2.2.

- Start the VM, choose Install Ubuntu (minimal is fine), create your username and password, and reboot when it finishes.

### 3.2 Install the packages (inside the VM)

sudo apt update
sudo apt install -y openssh-server mininet openvswitch-switch iperf netcat-openbsd curl git unzip python3-matplotlib
sudo systemctl enable --now ssh
sudo mn --test pingall

The last command should end with 0% dropped. If apt says it is waiting for a lock, Ubuntu is running its automatic updater, so wait a few minutes. If the mininet package cannot be found, install it from source:

git clone https://github.com/mininet/mininet
sudo PYTHON=python3 mininet/util/install.sh -nv

### 3.3 Ryu environment (inside the VM)

curl -LsSf https://astral.sh/uv/install.sh | sh
source ~/.local/bin/env
uv venv --python 3.9 ~/ryu-env
source ~/ryu-env/bin/activate
uv pip install 'setuptools<58' wheel pbr
uv pip install --no-build-isolation ryu==4.34 eventlet==0.30.2
ryu-manager --version

The last line must print a version number.

### 3.4 Connect VS Code to the VM

First test from Windows PowerShell (type yes at the fingerprint question, then your VM password):

ssh -p 2222 <your-username>@127.0.0.1

Then in VS Code press Ctrl+Shift+P, choose Remote-SSH: Open SSH Configuration File, pick the first file, and add:

Host exam-vm
  HostName 127.0.0.1
  Port 2222
  User <your-username>

HostName must be 127.0.0.1; the name after Host can be anything. Then press Ctrl+Shift+P, choose Remote-SSH: Connect to Host, pick exam-vm, and choose Linux if asked. The bottom-left corner shows SSH: exam-vm when connected.

### 3.5 Copy the project into the VM

In Windows PowerShell (put quotes around the path if it contains spaces):

scp C:\\Users\\<you>\\Downloads\\exam-sdn.zip exam-vm:~/

Then in the VM terminal:

cd ~
unzip exam-sdn.zip
cd ~/exam-sdn
ls

You should see README.md, client, common, controller, data, requirements.txt, results, server, tests, topology and web. If the zip unpacked into a nested exam-sdn folder, go into the one that contains server and client. All commands below run from that folder. To allow your own student ID, add it to data/roster.txt.

## 4. Running the project, in order

Three rules prevent most problems:

- Order: run sudo mn -c first, then start the controller, then run experiments. The command mn -c also kills a running controller.

- Start the controller with --observe-links, in a terminal where the Ryu environment is activated, and never with sudo.

- Anything that starts Mininet needs sudo python3. Long jobs are safest in the VM terminal or started with nohup.

### 4.1 Test the exam application alone (no Mininet)

Run in the VM terminal (the ampersand does not work in PowerShell):

cd ~/exam-sdn
python3 server/exam_server.py --port 5055 --expected 2 --duration 30 &
python3 client/student_client.py --port 5055 --id S001 --name 'Test One' --think 0.3 &
python3 client/student_client.py --port 5055 --id S002 --name 'Test Two' --think 0.3

Expected: both students log in, EXAM START, a SUBMIT line with a score for each, EXAM END.

### 4.2 Answer the questions yourself (interactive mode)

# terminal 1
python3 server/exam_server.py --port 5055 --expected 1 --duration 60
# terminal 2
python3 client/student_client.py --port 5055 --id S001 --name 'Your Name' --interactive

Type the option number (1 to 4) and press Enter for each question. The result is printed after the last question or when the time runs out.

### 4.3 Start the controller

cd ~/exam-sdn
sudo mn -c
source ~/ryu-env/bin/activate
EXAM_MODE=sdn ryu-manager --observe-links controller/exam_controller.py

Expected: mode=sdn listener=udp/9999, then LINK_UP lines once Mininet starts. Leave this terminal running and open a second terminal for everything else. For the comparison runs, restart the controller as the plain baseline with EXAM_MODE=baseline (same command, different mode).

### 4.4 Quick Mininet test

cd ~/exam-sdn
sudo python3 tests/run_experiment.py --students 3 --bg 0 --duration 20 --label quick
cat results/quick/server.log
cat results/quick/results.csv
python3 tests/analyze.py results/quick

Expected: LOGIN lines for S001 to S003, EXAM START, scores, and a final line done -> .../results/quick. The repeated warnings about sch_htb quantum are harmless.

### 4.5 Unauthorized host test (blocking)

sudo python3 tests/run_experiment.py --students 3 --bg 0 --duration 20 --intruder --label intruder

Expected in the controller terminal: APP EVENT, then UNAUTHORIZED, then BLOCKED 10.0.0.11.

### 4.6 Web exam (15 minutes, 10 questions)

With the controller running:

cd ~/exam-sdn
sudo python3 tests/web_demo.py

Open Firefox inside the VM and go to http://10.0.0.1:8080 (student on h1). Use 10.0.0.2, 10.0.0.3 and 10.0.0.4 for more students at the same time. Enter an ID from data/roster.txt and a name, press Start exam, answer, and press Submit. The score and a per-question review appear after submission. Saved results are in results/web/results.csv. To use a Windows browser instead, run this in PowerShell and open http://localhost:8081:

ssh -N -L 8081:10.0.0.1:8080 exam-vm

Type exit at the mininet> prompt to stop.

### 4.7 Congestion experiment

With the controller in SDN mode:

sudo python3 tests/run_experiment.py --students 10 --bg 12 --label sdn_n10_bg12
python3 tests/analyze.py results/sdn_n10_bg12
cp results/sdn_events.csv results/sdn_events_cong.csv
cat results/sdn_events_cong.csv

Expected: a POLICE_ON row (the controller limiting background traffic). Then stop the controller, restart it with EXAM_MODE=baseline, and run the same experiment with --label baseline_n10_bg12 to get the comparison.

### 4.8 Link-failure experiment

sudo python3 tests/run_experiment.py --students 10 --redundant --think 4 --fail-at 10 --restore-after 20 --duration 90 --label sdn_fail
python3 tests/analyze.py results/sdn_fail
cp results/sdn_events.csv results/sdn_events_fail.csv

Expected: the runner prints link s1-s4 DOWN, the controller logs LINK_DOWN then REROUTE, and LINK_UP after the link is restored. Repeat with the baseline controller and --label baseline_fail. Ignore events after APP_END; they are Mininet shutting down.

### 4.9 Full evaluation set and charts

Stop your manual controller first; the script starts its own.

cd ~/exam-sdn
mv results/summary.csv results/summary_old.csv 2>/dev/null
sudo -v
sudo nohup python3 tests/run_matrix.py --quick < /dev/null > results/matrix.log 2>&1 &
tail -f results/matrix.log
python3 tests/plot.py

The quick set takes about 10 minutes. For the complete set (about 40 to 60 minutes):

sudo -v
sudo nohup python3 tests/run_matrix.py --students 5 10 20 --bg 0 12 --reps 2 --failure < /dev/null > results/matrix.log 2>&1 &
tail -f results/matrix.log
python3 tests/plot.py

Outputs: results/plots (PNG charts), results/summary_table.md and results/summary_table.csv. If it is interrupted, run the same command again and finished runs are skipped.

## 5. Command reference

| run_experiment.py option | Meaning |
| --- | --- |
| --students N | Number of students (spread over h1 to h8) |
| --bg M | UDP background load in Mbps from h10 to h12 (0 = none) |
| --duration S | Exam length in seconds (default 60) |
| --think S | Maximum think time per question (default 1) |
| --intruder | Host h11, not on the roster, tries to log in |
| --redundant | Adds the backup path s1-s5-s4 |
| --fail-at S, --restore-after S | Take link s1-s4 down S seconds after start, bring it back later (needs --redundant) |
| --bottleneck M | Bandwidth of the s1-s4 link (default 10) |
| --label NAME | Result folder name under results/ (required) |

Controller settings (environment variables): EXAM_MODE (sdn or baseline), EXAM_CONGESTION_MBPS (default 8), EXAM_BG_LIMIT_KBPS (default 2000), EXAM_POLICE_HOLD (default 15 seconds), EXAM_POLL (default 2 seconds).

## 6. Results so far

Single runs, 10 students. Latencies are per request, in milliseconds.

| Scenario | Mode | Mean | 95th percentile | Worst request | Jain fairness |
| --- | --- | --- | --- | --- | --- |
| 12 Mbps flood on the 10 Mbps link | Baseline | 1,822 | 3,115 | 24,574 | 0.77 |
| 12 Mbps flood on the 10 Mbps link | SDN | 36 | 101 | 385 | 0.94 |
| Link s1-s4 down for 20 s | Baseline | 1,645 | 29,433 | 36,769 | 0.99 |
| Link s1-s4 down for 20 s | SDN | 35 | 57 | 207 | 0.99 |

Control-plane evidence: POLICE_ON was logged at a peak of 14.1 Mbps, and on the link failure LINK_DOWN was followed by REROUTE within 4 ms, with traffic returning to the direct path after LINK_UP. All students completed in every run. These are single runs; the evaluation set in section 4.9 adds repeats and more conditions.

## 7. Rubric coverage

| Rubric item | Where in the project | Status |
| --- | --- | --- |
| Problem understanding and architecture (2) | Reports, architecture diagram, README | Done |
| TCP socket implementation (4) | protocol.py, exam_server.py, student_client.py, gateway.py | Done and tested |
| Mininet and SDN implementation (4) | exam_topo.py, exam_controller.py | Done, tested in the VM |
| Socket-SDN integration (3) | UDP events START, LOGIN, UNAUTHORIZED, END | Login and start events seen; blocking run still to be shown (section 4.5) |
| Initial testing and demo (2) | Sections 4.1 to 4.6 | Done |
| Complete application functionality (4) | Exam, timer, scoring, web page | Done |
| Dynamic SDN functionality (6) | Congestion limit, reroute, recovery, monitoring, blocking | Shown in single runs |
| Challenging scenario (4) | Congestion flood, link failure | Shown in single runs |
| Performance evaluation and comparison (5) | analyze.py, run_matrix.py, plot.py | Baseline vs SDN shown; repeats and charts still to run |
| Documentation, demo, viva (6) | README, reports, this guide | GitHub repository and final report still to do |

## 8. Troubleshooting

| Problem | Fix |
| --- | --- |
| apt waits for a lock | The automatic updater is running; wait, or run sudo systemctl stop unattended-upgrades |
| uv pip install ryu fails with get_script_args | Use the two-step install in section 3.3 (setuptools below 58, then --no-build-isolation) |
| ryu-manager: command not found | Run source ~/ryu-env/bin/activate in that terminal |
| RTNETLINK answers: File exists | Leftover Mininet state: run sudo mn -c, then restart the controller |
| Experiment hangs, server.log has no LOGIN lines | The controller is not running or has no topology: start it with --observe-links and wait for LINK_UP |
| Warnings: sch_htb quantum of class is big | Harmless |
| VS Code: could not resolve hostname | HostName in the SSH config must be 127.0.0.1 |
| Connection refused on 127.0.0.1:2222 | Run sudo systemctl enable --now ssh in the VM, check the port-forward rule, restart the VM fully |
| Dragging a file into VS Code opens it on Windows | Copy with scp instead (section 3.5) |
| Ampersand error in PowerShell | Those commands belong in the VM terminal, not PowerShell |
| Web page says the ID is not registered | Add the ID to data/roster.txt |
| VM freezes after some time | More RAM, a swap file, turn off screen sleep, or run without the desktop |

## 9. Evidence to keep for the demo and viva

- Controller terminal showing LINK_UP, APP EVENT, UNAUTHORIZED and BLOCKED lines.

- results/summary_table.md and the charts in results/plots.

- sdn_events files showing POLICE_ON, LINK_DOWN, REROUTE and LINK_UP.

- Web exam screenshots: login page, a question with the timer, and the result page.

- results/web/results.csv and results/quick/results.csv (names and scores).